Assignment 4 - Rock Paper Scissors

Elisha Gras (eg692)

Github Link:

# 1. Project Overview

This project implements a real-time Rock Paper Scissors gesture classifier running on an ESP32-S3 microcontroller. A Convolutional Neural Network (CNN) was trained on a custom dataset of hand gesture images captured using the ESP32's OV2640 camera module. The trained model was converted and deployed directly onto the ESP32, enabling on-device inference without any external compute.

The system captures a 96x96 grayscale image from the camera, preprocesses it to a 32x32 binary image using pixel sampling and thresholding, feeds it into the CNN and outputs a classification of rock, paper or scissors in real time.

# 2. Dataset

## 2.1 Image Capture

Images were captured directly from the ESP32-S3 camera using a custom MicroPython burst capture script. The camera was configured to capture 96x96 pixel grayscale images in BMP format. Approximately 171 raw images were captured per gesture class (rock, paper, scissors) totaling approximately 513 raw images.

Images were captured against a plain background, with some variation in hand distance and positioning to improve model robustness. Both hand-only and hand-with-body-in-frame captures were included.

## 2.2 Preprocessing

Raw 96x96 images were preprocessed on the laptop to match the exact preprocessing pipeline used by the ESP32 at inference time. This is critical for ensuring training and inference consistency. The preprocessing pipeline is:

- Sample every 3rd pixel from the 96x96 image to produce a 32x32 image (matching ESP32's resize_96x96_to_32x32_and_threshold function)

- Apply binary threshold at 128: pixels >= 128 become 255 (white), pixels < 128 become 0 (black)

- Apply data augmentation: horizontal flip, +10 degree rotation, -10 degree rotation

This produced approximately 684 processed images per class (171 raw x 4 augmentations) totaling 2,044 training images across the three classes.

# 3. CNN Architecture and Design

## 3.1 Model Architecture

A lightweight Convolutional Neural Network was designed to run within the memory and compute constraints of the ESP32-S3. The model takes a 32x32 single-channel (grayscale) image as input and outputs a probability distribution over 3 classes.

| Layer | Output Shape | Parameters | Activation |
| --- | --- | --- | --- |
| Input | (32, 32, 1) | - | - |
| Conv2D (8 filters, 3x3, stride 2) | (15, 15, 8) | 80 | ReLU |
| Conv2D (16 filters, 3x3, stride 2) | (7, 7, 16) | 1,168 | ReLU |
| Flatten | (784,) | 0 | - |
| Dense (32 units) | (32,) | 25,120 | ReLU |
| Dense (3 units) | (3,) | 99 | Softmax |

Total trainable parameters: 26,467 (103 KB). This compact architecture was chosen specifically to fit within the ESP32-S3's RAM and flash constraints.

## 3.2 Design Decisions

- Strided convolutions (stride=2) instead of MaxPooling layers: MaxPooling is not natively supported by the emlearn tmdl format used on the ESP32. Strided convolutions achieve the same spatial downsampling while remaining compatible with the deployment pipeline.

- Binary input images: Using threshold preprocessing produces binary (0 or 255) images, simplifying the feature space and making the model more robust to lighting variation.

- Lightweight architecture: The small number of parameters ensures the model loads and runs quickly on the microcontroller with limited RAM.

# 4. Training

The model was trained on a laptop using TensorFlow/Keras. Training configuration:

- Optimizer: Adam (learning rate: 0.001 default)

- Loss function: Categorical crossentropy

- Epochs: 20

- Batch size: 32

- Train/test split: 80% / 20% (1,635 training, 409 test)

The model achieved 98.0% test accuracy after 20 epochs, with training accuracy reaching 100%. The model converged rapidly, reaching above 95% validation accuracy by epoch 5.

# 5. Deployment Pipeline

The trained model was converted for deployment on the ESP32 using the following pipeline:

- Train CNN in Keras -> save as rps_model.h5

- Convert to TensorFlow Lite format ->rps_model.tflite (using TFLiteConverter)

- Convert to TinyMaix tmdl format -> prs_cnn.tmdl (using tflite2tmdl.py)

- Upload prs_cnn.tmdl to ESP32 filesystem via Thonny

The tmdl format is a compact binary model format supported by the emlearn_cnn_fp32 MicroPython library running on the ESP32. The final model size on the ESP32 is 103.8 KB flash and 11.0 KB RAM.

# 6. Real-Time Classification

The classify.py script runs on the ESP32 and performs real-time classification using the following loop:

- Capture 96x96 grayscale BMP image from camera

- Resize to 32x32 and apply threshold at 128 using resize_96x96_to_32x32_and_threshold()

- Strip BMP header using strip_bmp_header() to get raw 1024-byte pixel array

- Run model inference using emlearn_cnn.run()

- Apply smoothing: require 5 consecutive consistent frames at 90%+ confidence before printing

- Lock prediction for 3 seconds after each output to prevent flickering

Classification results are printed to the host PC via Thonny's serial shell in the format: >>> ROCK (99%)

# 7. Potential Improvements

## 7.1 More Training Data

The current dataset contains approximately 171 raw images per class. Increasing this to 500+ raw images per class, captured under a wider variety of lighting conditions, backgrounds, and hand positions, would significantly improve robustness. Particular focus on scissors gestures would help, as this class showed slightly lower real-time accuracy compared to rock and paper.

## 7.2 Better and Consistent Lighting

Lighting was a significant factor in classification accuracy. Images captured under inconsistent lighting (bright spots, shadows, backlighting) produced ambiguous binary images after thresholding. A controlled lighting setup with a diffused light source and plain background would produce more consistent training and inference images.

## 7.3 Deeper CNN Architecture

The current architecture is intentionally minimal to fit the ESP32's constraints. A deeper model with an additional convolutional layer or larger filter counts (e.g., 16/32/64 filters) could learn more discriminative features. Additionally, batch normalization layers could improve training stability. This would require careful optimization to stay within the ESP32's memory budget.

## 7.4 Adaptive Thresholding

The current fixed threshold of 128 may not be optimal under all lighting conditions. An adaptive thresholding approach (e.g., Otsu's method) that computes the threshold per-image based on the pixel intensity histogram could produce more consistent binary images across different environments, reducing the impact of lighting variation on classification accuracy.

# 8. Code Attribution

The following files were provided by the course instructor and used without modification:

- image_preprocessing.py - ESP32 image preprocessing functions (resize, threshold, strip header)

- tflite2tmdl.py - TFLite to TinyMaix tmdl conversion script

- tflite_reader.py - TFLite model parser (dependency of tflite2tmdl.py)

The following files were written by the student:

- train_cnn.py - CNN model definition and training

- convert_model.py- Keras to TFLite conversion

- reprocess.py - Dataset preprocessing and augmentation pipeline

- classify.py - Real-time classification on ESP32